AIoT PROJECT  ·  SEMESTER VII

FROSTMIND

An AI-driven cold-chain monitoring and intervention system

Project Documentation

Phase 7: Deployment, Documentation & Presentation

Prepared by	Team FROSTMIND

Document version	1.0 (27 October 2026)

Code base	github.com/UzairManjre/frostmind, branch feature/load-planner-models-4-5

Evaluation focus	Deployment, usability, documentation quality, demonstration, presentation, future scope, maintenance strategy

Contents

1. Executive summary	4

2. Problem statement and objectives	5

2.1 The problem	5

2.2 Objectives	5

2.3 Scope	5

3. System architecture	6

3.1 How one reading travels	6

3.2 Technology stack	6

4. Hardware	8

4.1 Bill of materials	8

4.2 Pin map and wiring	8

4.3 Power	9

5. Firmware (ESP32-S3)	10

5.1 Sensing and the on-device rule (Model 0)	10

5.2 Automatic AC pairing by thermal feedback	10

5.3 Bluetooth link to the phone	10

5.4 Status light strip	11

5.5 Bench tools	11

6. Phone bridge app	12

6.1 The six-step setup wizard	12

6.2 Live dashboard	12

6.3 Reliability on a moving truck	13

7. Backend	14

7.1 Data model	14

7.2 The ingest pipeline	14

7.3 Security	14

8. Admin panel	15

9. AI and machine-learning layer	17

9.1 Model 0: on-device rule	17

9.2 Model 1: anomaly detection	17

9.3 Model 2: root-cause classification (deferred)	18

9.4 Model 3: shelf life (Arrhenius kinetics)	18

9.5 Model 4: mixed-load setpoint	18

9.6 Model 5: which products can ship together	19

9.7 Data collection and cleaning	20

9.8 The local LLM layer	20

10. Deployment	21

10.1 Ports	21

10.2 Prerequisites	21

10.3 Installing on a fresh laptop	21

10.4 No IP lock: it works on any network	22

10.5 First-time phone setup	22

10.6 Configuration	22

10.7 Flashing the firmware	22

10.8 Troubleshooting	23

10.9 Path to production	23

11. Usability	24

11.1 Users and their goals	24

11.2 Design principles applied	24

11.3 Improvements found through real use	24

12. Testing and quality assurance	26

13. Demonstration plan	27

13.1 Before the demo	27

13.2 Ten-minute script	27

13.3 Fallbacks	27

14. Future scope	28

15. Maintenance strategy	29

15.1 Routine tasks	29

15.2 Model governance	29

15.3 Monitoring and incident response	30

15.4 Data and schema changes	30

15.5 Roles	30

16. Limitations	31

17. Conclusion	31

18. References	32

Appendix A. Repository structure	33

Appendix B. API endpoints	34

Appendix C. Command cheat sheet	35

Appendix D. Glossary	35

# 1. Executive summary

Refrigerated shipments of food and medicines fail quietly: a door left open, a cooling unit that stops, a load packed with products that should never travel together. Most low-cost data loggers only record the temperature. Someone reads the log after the damage is done.

FROSTMIND senses, acts and explains. A small ESP32 box on the truck reads temperature, humidity and the door. It decides on its own when something is wrong and fires an infrared command at the cooling unit. Through the driver's phone it streams every reading to a local server, where the AI layer computes how much shelf life each product has used, recommends the right temperature for mixed loads, and checks which products can share a compartment. A local language model then explains every incident in plain English. Nothing leaves the laptop: no cloud account is needed and no data is sent anywhere.

| What was delivered | Evidence |
| --- | --- |
| Working hardware | Seeed XIAO ESP32-S3 with DHT22, door reed switch, IR transmitter and an 11-LED status light strip. Flashed and verified on the bench (24.5 °C, door open/closed, strip in GRB order). |
| Complete software stack | Firmware, Android phone app (PWA), FastAPI backend and Next.js admin panel. 72 automated backend tests pass; the web app passes lint, type checks and a production build; the firmware compiles with zero warnings. |
| Seven AI components | Model 0 on-device rule (live), Model 1 anomaly detector (validated), Model 2 root-cause classifier (deferred, hardware-gated), Model 3 Arrhenius shelf life (live), Model 4 mixed-load setpoint: MAE 0.16 °C, Model 5 co-shipment compatibility: ROC-AUC 0.930, local LLM explanations (live). |
| One-command deployment | python start_frostmind.py runs the whole system on any laptop. It finds the network address itself, so nothing is tied to a fixed IP. |
| Usable by a driver | A 6-step guided setup, no typing of addresses or device IDs, plain-language error hints, AI explanations with a Do: action, and a light strip that shows the state of the trip at a glance. |
| Documentation and presentation | This document, a slide deck, wiring and BOM guide, firmware and phone READMEs, API docs at /docs, and the Model 4/5 technical report. |

Table 1. Phase 7 deliverables at a glance

The rest of this document follows the Phase 7 evaluation criteria. Sections 2 to 9 document the system itself. Section 10 covers deployment, 11 usability, 12 testing, 13 the demonstration plan, 14 future scope and 15 the maintenance strategy.

# 2. Problem statement and objectives

## 2.1 The problem

Cold-chain products have a temperature window they must stay inside, from vaccines (2 to 8 °C) to frozen food (−18 °C or colder). An excursion does not destroy a product instantly. Damage accumulates with time and temperature, so the questions that matter in practice are:

- Is something going wrong right now, and can it be corrected before the product is harmed?

- How much damage has already been done? A 20-minute excursion and a 6-hour one are very different.

- What should this particular load be kept at? A mixed load of apples, pears and lettuce has no single label temperature.

- Which products can safely share a compartment? Ripening bananas give off ethylene, which spoils lettuce; medicines must never travel with food.

- What happened, in words a manager or auditor can act on?

## 2.2 Objectives

- Monitor temperature, humidity and door state on a refrigerated load every 2 seconds with low-cost hardware.

- React automatically on the truck: correct the cooling unit over infrared without waiting for a human or an internet connection.

- Quantify product damage with a physics-based shelf-life model, per product.

- Use machine learning where real data supports it: mixed-load setpoints (Model 4) and co-shipment compatibility (Model 5).

- Explain every incident in plain language with a local LLM that can never take a physical action itself.

- Make it deployable on any laptop and usable by a driver with no technical training.

## 2.3 Scope

In scope: one truck or compartment per device, an Android phone as the network bridge, a laptop as the server, and a room air conditioner as the stand-in for a reefer unit during testing. Out of scope for this phase: cellular or satellite connectivity, a cloud deployment, iOS support (Safari does not implement Web Bluetooth), and certification for regulated pharmaceutical decisions.

# 3. System architecture

FROSTMIND has four tiers. The edge device senses and acts. The phone bridge carries data between Bluetooth and the network, because the ESP32 box has no internet radio of its own. The backend stores readings and runs the AI models. The admin panel is the fleet manager's view. A local LLM sits beside the backend and writes the explanations.

Figure 1. System architecture: edge device, phone bridge, backend, admin panel and the local LLM

## 3.1 How one reading travels

- The ESP32 samples the DHT22 and the reed switch every 2 seconds and applies the Model 0 rule. If the rule trips and an AC protocol is paired, it fires the IR command immediately (5-minute cooldown).

- The reading goes to the phone over Bluetooth Low Energy as one line of JSON.

- The phone adds its GPS position and puts the reading in an offline queue (IndexedDB), so nothing is lost when the network drops.

- The sync manager uploads queued readings to POST /ingest over HTTPS, authenticated with the device's own key.

- The backend stores the readings, updates Model 3 shelf life for every product, checks the Model 4 setpoint, and raises alerts.

- For the newest alert of each type, a background task asks the local LLM for an explanation and stores it on the alert.

- The phone dashboard (every 10 s) and the admin panel show the readings, shelf life, alerts and AI explanations. The phone also sends the shelf life and alert level back to the box's light strip.

## 3.2 Technology stack

| Tier | Technology | Why |
| --- | --- | --- |
| Edge device | Seeed XIAO ESP32-S3, Arduino core 3.3.11, DHT sensor library, IRremoteESP8266 2.9.0, ArduinoJson 7, bundled BLE, RMT for the LED strip | Small, cheap, Bluetooth built in; IRremoteESP8266 speaks about 65 AC protocols |
| Phone bridge | Plain HTML/JS ES modules (no build step), Web Bluetooth, IndexedDB, service worker, Geolocation, Wake Lock | Runs in Chrome on any Android phone with nothing to install; works offline |
| Backend | Python 3.12, FastAPI, SQLAlchemy 2, SQLite, Pydantic 2, PyJWT, scikit-learn, NumPy/SciPy | Fast to build, typed API with automatic docs, ORM ready for PostgreSQL |
| Admin panel | Next.js 16, React 19, TypeScript, dependency-free CSS design system | Modern web UI; served from the same laptop |
| AI layer | scikit-learn (Random Forest, Histogram Gradient Boosting, Isolation Forest), physics models in NumPy/SciPy | Interpretable models that run on a laptop CPU |
| LLM | Ollama with a quantised Gemma instruction model (QAT, 4-bit) | Runs locally: no cloud API, no per-call cost, no data leaving the machine |

Table 2. Technology stack

# 4. Hardware

## 4.1 Bill of materials

| Part | Specification | Qty | Purpose |
| --- | --- | --- | --- |
| Seeed XIAO ESP32-S3 | ESP32-S3R8, 8 MB PSRAM, USB-C, BLE 5 | 1 | Brain of the device; powered and flashed over USB-C |
| DHT22 (AM2302) module | 3-pin single-wire, ±0.5 °C, 0-100 % RH | 1 | Temperature and humidity |
| MC-38 reed switch | 2-wire, normally open, with magnet | 1 | Door open/closed |
| IR LED | 940 nm, 5 mm | 1 | Sends AC commands (transmit only, no IR receiver in this build) |
| WS2812 / WS281x LED strip | 5 V addressable RGB, GRB colour order, 11 LEDs | 1 | Status light: pairing, shelf life, alerts |
| Resistor (optional) | 100-220 Ω | 1 | IR LED current limit |
| Breadboard, jumper wires | - | - | Connections; one ground rail shared by all parts |
| USB-C cable | Data-capable | 1 | Power and flashing (a charge-only cable cannot flash) |

Table 3. Bill of materials

## 4.2 Pin map and wiring

Important Wire by the D-label printed next to each hole. On the XIAO ESP32-S3, Dn is not GPIO n: D3 = GPIO4, D4 = GPIO5, D5 = GPIO6, D6 = GPIO43, D8 = GPIO7. The XIAO ESP32-C3 looks identical but maps differently, so its pinout does not apply. This was the cause of an early 'IR LED never lights' problem.

Figure 2. Wiring on the XIAO ESP32-S3

| XIAO pad | GPIO | Function | Connects to |
| --- | --- | --- | --- |
| 3V3 | - | 3.3 V supply | DHT22 VCC |
| 5V | - | 5 V from USB | LED strip 5V |
| GND | - | Ground | Breadboard GND rail: DHT GND, reed switch, IR LED short leg, strip GND |
| D8 | GPIO7 | Single-wire data | DHT22 DATA |
| D4 | GPIO5 | Input, internal pull-up | Reed switch (other leg to GND). HIGH = door open |
| D5 | GPIO6 | Digital output | IR LED long leg (anode) |
| D3 | GPIO4 | Output (RMT peripheral) | LED strip DIN (use the strip's input end) |
| D0-D2, D6, D7, D9, D10 | - | Free | Leave empty (D6/D7 are the UART0 pins) |

Table 4. Pin map (constants in firmware/frostmind_main/frostmind_main.ino)

## 4.3 Power

Everything runs from the USB-C port. The firmware caps the strip's brightness (80/255 after gamma correction), so 11 LEDs stay well under the USB budget. On a truck the same 5 V would come from a USB adapter on the vehicle supply, with a battery backup listed as future work.

# 5. Firmware (ESP32-S3)

One sketch, firmware/frostmind_main/frostmind_main.ino, has five responsibilities: sensing and the on-device rule, AC control, automatic AC pairing, the Bluetooth link to the phone, and the status light strip.

## 5.1 Sensing and the on-device rule (Model 0)

Every 2 seconds the firmware reads the DHT22 and the reed switch, keeps the last 6 temperatures, and computes the least-squares slope in °C per minute. The rule is deliberately simple and explainable:

rate      = least-squares slope of the last 6 samples      (°C/min)

tempAlert = |rate| > 3.0   OR   |temp - category setpoint| > 3.0 °C

doorAlert = door open for more than 60 s

alert     = tempAlert OR doorAlert

On an alert, if an AC protocol is paired, the device sends power on, cool, fan high over IR, then waits at least 5 minutes before it can fire again. Every 5 minutes it also re-sends the last command (the heartbeat). With no IR receiver it cannot confirm that a command landed, so re-asserting the intended state is the only mitigation. The heartbeat also shows the phone that the IR path is alive.

## 5.2 Automatic AC pairing by thermal feedback

Normally a universal remote learns a protocol by capturing the original remote with an IR receiver. This build has none, so FROSTMIND uses the room itself as the feedback signal:

- Measure the baseline temperature slope for about 60 s without touching the AC.

- For each candidate protocol (a shortlist of common brands first, Samsung first, then all of the library's roughly 65 protocols), send power on + cool and measure the slope for about 100 s.

- If the room cools noticeably faster than the baseline, send power off on the same protocol and check that the trend reverses within about 45 s. A coincidental drop is rejected here.

- On success, save the protocol in flash (NVS), so it survives reboots. If nothing matches, a manual mode lets a person step through candidates from the phone or the serial console.

## 5.3 Bluetooth link to the phone

The device is a BLE peripheral using the Nordic UART Service pattern. Messages are newline-delimited JSON in both directions. The phone writes commands in 20-byte chunks, which the firmware reassembles before parsing.

| Direction | Messages |
| --- | --- |
| Device to phone, every 2 s | {"ts", "temp", "hum", "door", "roc", "alert", "ac_commanded", "ac_degrees", "ac_power", "ac_last_cmd_ms"} |
| Device to phone, on change | {"type":"pairing_status", "state": scanning ¦ matched ¦ no_match ¦ idle, "candidate_index", "total"} |
| Phone to device | set_category, start_pairing, pairing_status, manual_pair, manual_confirm, led (shelf life left + alert level for the strip), identify (5 s rainbow) |

Table 5. BLE contract summary (full contract: firmware/frostmind_main/README.md)

## 5.4 Status light strip

An 11-LED WS2812 strip gives the driver the state of the trip at a glance. It is driven straight from the ESP32's RMT peripheral, so no extra library is needed and the timing is done in hardware. Frames are drawn in floating point at 60 fps, lights move at sub-pixel positions, every change cross-fades over 450 ms, and the output is gamma-corrected and dithered.

The strip follows a workflow and shows one look at a time. Nothing is layered on top of anything else, and short animations queue up instead of cutting each other off. Problems are only shown while a phone is connected and a trip is live: with no phone connected nobody would see them, and during setup no product or setpoint has been chosen yet.

Figure 3. The light strip's workflow states

## 5.5 Bench tools

scripts/esp_console.py is a live serial console that formats each reading and passes keys to the firmware:

| Key | Action |
| --- | --- |
| x | IR LED test: 3 s of flashing (view it through a phone camera; IR is invisible to the eye) |
| g | Light strip colour test: red, green, blue, white for 1 s each (checks the GRB colour order) |
| d | Light strip demo: the whole workflow in order, 4.5 s per look, with captions |
| a | Start automatic AC pairing |
| l, n, digits + Enter, y | Manual pairing: list protocols, next candidate, jump to an index, confirm and save |
| +, -, f, p, r, c | AC temperature up/down, fan, power, resend, clear the saved pairing |

Table 6. Serial console keys

# 6. Phone bridge app

The phone app (phone-bridge/) is a Progressive Web App opened in Chrome on Android. It needs HTTPS for Web Bluetooth, which the launcher provides with a self-signed certificate. There is nothing to install from an app store.

## 6.1 The six-step setup wizard

| # | Step | What happens |
| --- | --- | --- |
| 1 | Pair phone to device | Chrome's Bluetooth picker lists FROSTMIND boxes (matched on the advertised service or the name). Errors come back as instructions, e.g. 'Bluetooth is off on this phone, turn it on and tap Pair again'. |
| 2 | Register device | The device ID is read from the Bluetooth connection, not typed. The device and its truck are registered together, and the device receives its own key. |
| 3 | What are you shipping? | Add products and kilograms (search or presets), then Check load (AI). Model 5 says what can travel together and why not; Model 4 gives each compartment's temperature. Create the shipment for this truck's compartment. The box receives the nearest preset and ticks the 'product' step on its light strip. |
| 4 | Auto-pair the AC | Starts the thermal-feedback sweep and shows its progress, with manual pairing as a fallback. |
| 5 | Confirm live data | Waits for real readings before continuing (device to phone). Flash the device's lights checks the other direction: the box plays a rainbow. |
| 6 | Go live | Starts the upload queue, GPS and the live dashboard. The light strip plays the 'go live' sweep and shows the shelf-life bar. |

Table 7. Setup wizard

## 6.2 Live dashboard

- Tiles: temperature, Bluetooth state, humidity, door, readings waiting to sync, shelf life used, GPS, the AC's last commanded setting and when the last IR command was sent.

- AI panel, fed by GET /shipments/{id}/live every 10 s: Model 4's recommended setpoint against the actual temperature, shelf life per product (Model 3), and the latest alerts with the LLM's assessment and a Do: action.

- Light strip panel: what the box should be showing right now, an Identify button and a colour legend.

- Reconnect Bluetooth, and Mark delivered / end shipment.

Figure 4. Left: setup step 3, the AI load check splits products into compartments with setpoints. Right: the dashboard AI panel with setpoint vs actual, per-product shelf life and an AI-explained alert

## 6.3 Reliability on a moving truck

- Offline queue: readings are stored in IndexedDB first and uploaded when the backend is reachable, so a dead zone loses nothing.

- Auto-reconnect: after a Bluetooth drop the app retries with back-off (1, 2, 4, 8, 15, then every 30 s).

- Screen wake lock: keeps the tab alive while the dashboard is open. A browser cannot hold Bluetooth in the background; this limit is documented, not hidden.

- Serialised commands: Bluetooth writes are queued so two commands never collide.

- Time handling: server timestamps are treated as UTC, so alert times are correct in India (UTC+5:30).

# 7. Backend

The backend (backend/) is a FastAPI application over SQLAlchemy and SQLite. It exposes a typed REST API with interactive documentation at /docs. The main endpoint groups are /auth, /devices (registration, pairing status), /trucks, /shipments (create, update, readings, alerts, live feed, CSV report, compliance report), /ingest, /alerts, /analytics/fleet, /products, /planner/plan, /ai/status, /ai/llm/test and /health; the full list is in Appendix B.

## 7.1 Data model

| Entity | Holds |
| --- | --- |
| Device | MAC/ID, name, provisioning key, last seen, AC pairing state and protocol |
| Truck | Label, plate, assigned device |
| Shipment | Category or product list (mixed load), status, start/end, Model 4 setpoint, safe band and method, Model 3 shelf life (shipment level and per product), threshold overrides |
| SensorReading | Timestamp (server receipt time), temperature, humidity, door, rate of change, device alert flag, anomaly score, latitude/longitude |
| Alert | Type, severity, message, acknowledged, LLM assessment, recommended action and narrative |
| User | Admin accounts (hashed passwords) for the panel |

Table 8. Database entities

Schema changes are additive: on start-up, ensure_columns() adds any new nullable column to an existing database, so updating the code never requires deleting data.

## 7.2 The ingest pipeline

- Authenticate the device with its X-Device-Key and check that the shipment belongs to it.

- Store the batch, timestamped at the server's receipt time (the device clock is uptime, not wall-clock).

- Update Model 3 shelf life for every product; the most perishable product drives the shipment's number.

- Raise alerts: an anomaly (device rule tripped, or more than 3 °C from Model 4's recommended setpoint on a mixed load), door open too long, and shelf-life thresholds (80 % used, then 100 %).

- Queue an LLM explanation for the newest alert of each type, newest first, so a backlog never queues minutes of LLM work.

## 7.3 Security

- Admin endpoints need a JWT from POST /auth/login (8-hour expiry by default).

- Device endpoints (/ingest, /shipments/{id}/live) need that device's own key, issued at registration.

- Setup endpoints (device registration, trucks, shipment creation, the planner) are open so the phone wizard works without an admin login. Locking them to a fleet enrolment code is on the roadmap.

- Secrets and the default admin account are environment variables (Section 10.6) and must be changed before any real pilot.

- The LLM is read-only: it writes text onto alerts and can never trigger a device or change a shipment.

# 8. Admin panel

The admin panel (web/) is the fleet manager's view: a Next.js application with a consistent design system. It works from the laptop or from any device on the same network.

| Page | What it shows |
| --- | --- |
| Fleet (home) | Every truck and device with its current shipment, status and last-seen time; refreshes every 5 s |
| Shipment detail | Live temperature and humidity chart with flagged readings and door-open shading, the mixed-load AI plan (Model 4 setpoint, per-product shelf life), alerts with AI analysis, CSV export and an LLM compliance report |
| Alerts | All alerts with severity, acknowledgement and the LLM's assessment and recommended action |
| Analytics | Fleet trends over a chosen window: alerts by type, and trucks ranked by alerts |
| Reports | Shipment audit trails: each shipment's record for download |
| Load Planner | Pick products and quantities, get compartments, setpoints, a compatibility matrix with reasons, and create the shipment for a truck |
| AI Models | Live status and measured scores of every AI component, where each is used, usage counts, and a one-click LLM test |

Table 9. Admin panel pages

Figure 5. Shipment page for a mixed load: Model 4 setpoint, per-product shelf life and the live chart

Figure 6. AI Models page: status, scores and a one-click LLM test

# 9. AI and machine-learning layer

FROSTMIND stages its models to the data that actually exists. Rules and physics run where there is no data to learn from yet; machine learning is used where real labelled data exists; the LLM only explains. Only deterministic logic can trigger a physical action.

| Component | Type | Data and result | Status |
| --- | --- | --- | --- |
| Model 0: alert rule | Deterministic rule on the ESP32 | Rate of change, setpoint deviation, door time. No training needed. | Live, fires IR |
| Model 1: anomaly detector | Isolation Forest (unsupervised) | Validated on 4 datasets, ROC-AUC 0.77 to 0.91 | Validated, switched off by design |
| Model 2: root cause | Supervised classifier (planned) | Needs AC current draw (INA219), not sourced | Deferred |
| Model 3: shelf life | Arrhenius kinetics (physics) | 6 categories plus a Q10 model per product | Live, every reading |
| Model 4: mixed-load setpoint | Physics optimiser + gradient-boosting student | MAE 0.16 °C, 93.6 % within ±0.5 °C on unseen products | Live (guarded) |
| Model 5: compatibility | Random Forest over 13 pair features | ROC-AUC 0.930, MCC 0.717 on unseen products | Live + policy vetoes |
| LLM layer | Local Gemma via Ollama | Explains alerts, writes compliance reports | Live, read-only |

Table 10. The AI components

## 9.1 Model 0: on-device rule

Described in Section 5.1. It is the only component allowed to act physically, because it is fully predictable and needs no network.

## 9.2 Model 1: anomaly detection

An Isolation Forest (contamination=0.02) trained only on normal data, using rolling mean, rolling standard deviation, rate of change and door-open duration over 10 readings. The decision threshold is chosen on one half of the held-out data and the metrics are reported on the untouched other half.

| Dataset | Real or simulated | Accuracy | ROC-AUC | F1 |
| --- | --- | --- | --- | --- |
| NAB machine temperature | Real | 85.6 % | 0.770 | 0.450 |
| Kaggle cold-chain milk (92 sessions) | Real | 83.8 % | 0.851 | 0.751 |
| Kaggle shipment failure (semi-supervised) | Simulated | 74.2 % | 0.809 | 0.719 |
| Kaggle shipment failure (supervised Random Forest) | Simulated | 82.3 % | 0.913 | 0.661 |

Table 11. Model 1 validation results

It is switched off against live data for two stated reasons: the shipment models use trip-level features, not the 2-second stream; and each model is calibrated to one temperature band, so frozen food would look anomalous to a milk-trained model. The backend has a ready seam (app/ml.py) that activates it automatically once a model trained on FROSTMIND's own logged sessions exists.

## 9.3 Model 2: root-cause classification (deferred)

Telling a compressor fault from an open door or a power cut needs the AC's current draw (an INA219 sensor). That sensor was not sourced, and no public dataset exists to simulate it credibly, so no model was faked in its place.

## 9.4 Model 3: shelf life (Arrhenius kinetics)

k(T) = A · exp(−Ea / (R·T))          rate constant, T in kelvin

used fraction = ∫ k(T(t)) dt  /  ( k(T_ref) × rated shelf life )

| Category | Reference temp | Rated shelf life | Ea (kJ/mol) |
| --- | --- | --- | --- |
| Generic refrigerated vaccine | 5 °C | ~6 months | 83 |
| Pasteurised milk | 4 °C | ~2 weeks | 65 |
| Fresh produce | 3 °C | ~10 days | 55 |
| Chilled raw meat/poultry | 2 °C | ~5 days | 60 |
| Frozen food | −18 °C | ~6 months | 45 |
| Fresh chilled seafood | 0 °C | ~2 days | 70 |

Table 12. Model 3 product categories (illustrative literature values, not certified constants)

For mixed loads each product gets its own shelf-life curve (a Q10 integral from the product catalog), and the most perishable product drives the shipment. Example from the real code: a vaccine shipment held at 5 °C for 48 h with a 6-hour excursion to 12 °C used 1.31 % of its shelf life.

## 9.5 Model 4: mixed-load setpoint

Question: one compartment holds several products, so what single temperature is best for all of them? A physics teacher minimises the expected quality loss across the products. It accounts for each product's decay rate (Q10), chilling injury below its safe minimum, freezing, and the truck's real temperature wobble σ (measured from real reefer logs), subject to safety constraints (for example medicines 2 to 8 °C, ready-to-eat food at or below 5 °C). A gradient-boosting student learned from 16,000 simulated loads and was tested on 4,000 loads made only of products it never saw.

| Method | MAE | Within ±0.5 °C | Note |
| --- | --- | --- | --- |
| Model 4 student (served) | 0.16 °C | 93.6 % | R² 0.999; served only if it agrees with physics to 0.5 °C |
| Rule baselines (midpoint, weighted mean, max) | 1.3-1.4 °C | much lower | Ignore chilling injury and uncertainty |

Table 13. Model 4 results on unseen products

Figure 7. Model 4: student prediction against the physics optimum on unseen products

## 9.6 Model 5: which products can ship together

A Random Forest scores every pair of products on 13 symmetric physical features: temperature-range overlap, humidity overlap, ethylene producer × sensitivity, chilling and freezing risk, odour transfer and more. Labels come from the USDA expert compatibility groups (McGregor, Agricultural Handbook 668), 5,671 produce pairs. It is evaluated on products it never saw, over 30 random splits. Regulatory vetoes are added on top (medicines never with food; raw meat apart from ready-to-eat food), and the order is split into compartments with DSatur graph colouring. Every 'keep apart' comes with a plain-English reason.

| Method | ROC-AUC | Accuracy | F1 | MCC |
| --- | --- | --- | --- | --- |
| Model 5 Random Forest | 0.930 | 86.7 % | 0.822 | 0.717 |
| Best hand-written rule | 0.84 | - | - | - |

Table 14. Model 5 results on unseen products (mean of 30 product-disjoint splits)

Figure 8. Model 5 ROC and precision-recall curves

## 9.7 Data collection and cleaning

Models 4 and 5 needed data the project did not have, so a product catalog was built from public sources: Engineering ToolBox storage conditions, USDA Handbook 66 (via UMaine Bulletin 4135), the McGregor compatibility groups (FAO reproduction of USDA AH-668), and a cited table for dairy, meat, seafood, frozen food and medicines (USDA FSIS, FDA Food Code, CDC, WHO/EU GDP). Each source was saved as a raw snapshot with its SHA-256 hash. The cleaning pipeline logs every step and its row counts: table extraction and repair, unit conversion (°F to °C), parsing of ranges and storage life, ethylene normalisation, chilling-sensitivity inference, name harmonisation and de-duplication, a cross-source agreement check, gap filling with flags, and validation. The result is 155 products. Full details are in FROSTMIND_Model4_Model5_Submission.docx.

## 9.8 The local LLM layer

|  | Role A: alert reasoning | Role B: compliance report |
| --- | --- | --- |
| Trigger | Automatically, for the newest alert of each type | On demand from the admin panel |
| Input | The alert, Model 3 shelf life, Model 4 setpoint, products, recent readings and alerts | The whole shipment history |
| Output | JSON: assessment, recommended action, narrative | A prose incident narrative for an auditor |
| Shown on | Phone AI panel and the admin Alerts and Shipment pages | Admin Shipment page |

Table 15. LLM roles

Reliability rules: every LLM call has a 45-second timeout and returns nothing on any failure, so the deterministic alert always stands on its own. The backend picks the configured model if it is installed, otherwise the smallest installed instruction model. In the end-to-end test an explanation reached the phone 7 seconds after the alert.

# 10. Deployment

The whole system runs on one laptop. The driver's phone and any admin browser connect to it over the local Wi-Fi, and the ESP32 talks only to the phone, over Bluetooth.

Figure 9. Deployment topology and ports

## 10.1 Ports

| Port | Protocol | Service |
| --- | --- | --- |
| 8000 | HTTPS | Backend for the phone (HTTPS, so the HTTPS phone page may call it) |
| 8001 | HTTP | Same backend for the admin panel (no certificate prompts) |
| 5500 | HTTPS | Phone app static files (serve_https.py) |
| 3000 | HTTP | Admin panel (Next.js) |
| 11434 | HTTP | Ollama, local only |

Table 16. Network ports

## 10.2 Prerequisites

- Windows, macOS or Linux laptop with Python 3.12 and Node.js 20 or newer.

- Ollama with the Gemma model (or any small instruction model; the backend picks one automatically).

- Arduino CLI with the esp32 core 3.3.11 and the libraries DHT sensor library, Adafruit Unified Sensor, IRremoteESP8266 2.9.0 and ArduinoJson 7 (only to flash the firmware).

- An Android phone with Chrome, Bluetooth and Location on, on the same Wi-Fi as the laptop.

## 10.3 Installing on a fresh laptop

git clone https://github.com/UzairManjre/frostmind.git && cd frostmind

python -m venv .venv

.venv\Scripts\pip install -r backend/requirements.txt -r scripts/requirements.txt

# build the catalog and train the models (the .pkl files are not in Git)

.venv\Scripts\python scripts/train_all.py

cd web && npm install && cd ..

ollama pull hf.co/unsloth/gemma-4-E4B-it-qat-GGUF:UD-Q4_K_XL

.venv\Scripts\python start_frostmind.py

The launcher prints the exact addresses to open, for example https://192.168.1.20:5500 for the phone and http://localhost:3000 for the admin panel. Ctrl+C stops every server.

## 10.4 No IP lock: it works on any network

A hard-coded address breaks the moment the laptop joins a different Wi-Fi. Every part of FROSTMIND works out its addresses at run time instead:

- Launcher: detects the laptop's current LAN addresses on every start.

- Certificate: generated to cover every current address plus localhost, and reused while it still covers them and has more than 30 days left. A phone accepts it once, not after every restart.

- Phone app: calls the backend on the same host it was opened from. A saved address from an old network is ignored automatically.

- Admin panel: calls the backend on the same host the page was opened from, and the dev server allows the laptop's current addresses, so it also works from another device.

## 10.5 First-time phone setup

- On the phone, open https://<laptop-IP>:8000/health, tap Advanced, then Proceed (accepts the certificate for the API).

- Open https://<laptop-IP>:5500, tap Advanced, then Proceed. Optionally add it to the home screen.

- Follow the setup wizard (Section 6.1). If a previous trip is still saved, use Settings, then Reset wizard.

## 10.6 Configuration

| Environment variable | Default | Purpose |
| --- | --- | --- |
| FROSTMIND_DATABASE_URL | sqlite:///backend/frostmind.db | Database (any SQLAlchemy URL, e.g. PostgreSQL) |
| FROSTMIND_JWT_SECRET | development value | Signs admin tokens. Change before a pilot. |
| FROSTMIND_JWT_EXPIRE_MINUTES | 480 | Admin session length |
| FROSTMIND_ADMIN_EMAIL / _PASSWORD | development account | First admin account. Change before a pilot. |
| FROSTMIND_CORS_ORIGINS |  | Allowed browser origins |
| FROSTMIND_OLLAMA_URL | http://localhost:11434/api/chat | LLM endpoint |
| FROSTMIND_OLLAMA_MODEL | gemma-4 E4B QAT (4-bit) | Preferred model |
| FROSTMIND_OLLAMA_TIMEOUT_SECONDS | 45 | LLM timeout |

Table 17. Backend configuration

## 10.7 Flashing the firmware

arduino-cli board list                  # find the port, e.g. COM5

set FQBN=esp32:esp32:esp32s3:CDCOnBoot=cdc

arduino-cli compile --fqbn %FQBN% firmware/frostmind_main

arduino-cli upload -p COM5 --fqbn %FQBN% firmware/frostmind_main

.venv\Scripts\python scripts/esp_console.py   # live console

Note CDCOnBoot=cdc is required. Without it, flashing succeeds but the sketch's serial output goes to unused UART pins and the console stays silent after the boot banner.

## 10.8 Troubleshooting

| Symptom | Fix |
| --- | --- |
| The box does not appear in the phone's Bluetooth picker | Turn on Bluetooth and Location; allow Chrome 'Nearby devices'; close other FROSTMIND tabs (one connection at a time); power-cycle the box. |
| The phone shows certificate or network errors | Accept the certificate at both :8000/health and :5500. Check that the phone and the laptop are on the same Wi-Fi. |
| The admin panel loads but buttons do nothing from another device | Restart the launcher on the current network (it allows the current addresses at start). |
| Temperature and humidity read 0.00 | A loose ground or power wire: check the DHT's 3V3 and the ground rail. (Found on the bench: a probe sketch showed the sensor itself was fine.) |
| Light strip shows wrong colours | Run g: the first colour must be red. If not, the strip isn't GRB. |
| Last LEDs of the strip stay dark | Set STATUS_LED_COUNT to the strip's LED count and reflash. |
| The IR LED never flickers on camera | The long leg must be in pad D5 (not D6). |
| No AI explanations | Check Ollama is running; the AI Models page and /health show the LLM status, and Test LLM gives a real reply. |

Table 18. Troubleshooting

## 10.9 Path to production

The laptop deployment is right for a pilot and a demonstration. For a real fleet: switch to PostgreSQL (one connection string, since all queries go through the ORM); serve the API on a real domain with a Let's Encrypt certificate; containerise the backend and admin panel with Docker; move LLM inference to a small GPU server; and add log aggregation and uptime alerting. Section 14 lists these on the roadmap.

# 11. Usability

## 11.1 Users and their goals

| User | Goal | Where |
| --- | --- | --- |
| Driver | Set the truck up in a few minutes and not have to think about it again | Phone app and the light strip on the box |
| Fleet manager | See every truck, act on problems, prove compliance | Admin panel |
| Technician | Install and check a box | Wiring guide, serial console, test keys |

Table 19. Users

## 11.2 Design principles applied

- Nothing to type: the device ID comes from the Bluetooth connection and the server address from the page URL. The driver never enters an IP or an ID.

- Guided and verified: a 6-step wizard that refuses to finish on faith. It waits for real readings, and the Identify button proves that the phone can reach the box.

- Errors as instructions: Bluetooth failures explain what to do next ('Turn on Bluetooth and Location, then tap Pair again'), not the browser's raw error.

- Plain-language AI: each alert's explanation ends with a Do: action a driver can follow.

- Glanceable status: the light strip follows the trip and shows one look at a time. It stays calm when nobody is connected, and on idle it shows just the shelf life.

- Consistent meaning: the same colours mean the same thing on the strip, the phone and the admin panel: mint is good, amber is a warning, coral is a problem, and the blue-violet-pink gradient is the AI.

- Forgiving: an offline queue, auto-reconnect, a wake lock, and a reset option for a new trip.

- Accessible: status is never shown by colour alone (the phone states in words what the strip shows), with large touch targets, high-contrast dark UI and capped LED brightness.

## 11.3 Improvements found through real use

The system was used end to end on real hardware, and every problem found was fixed and documented:

| Problem found | Fix |
| --- | --- |
| The box never appeared in Chrome's Bluetooth picker | Advertise the service UUID in the advertisement, with a name-prefix filter as fallback |
| Phone commands silently ignored by the box | The firmware reassembles 20-byte Bluetooth chunks into whole lines |
| The phone app was only a monitor; no multi-product loads, no AI visible | Added the 'What are you shipping?' step with the AI load check, and the AI panel with explanations |
| Admin panel dead when opened from another device | Allow the laptop's current addresses; the API base follows the page host |
| Hard-coded IP broke on a new network | No-IP-lock design (Section 10.4) |
| Certificate warning after every restart | Reuse the certificate while it is still valid |
| Alert times 5.5 hours off on the phone | Server timestamps are parsed as UTC |
| Two Bluetooth commands at once collided | Commands are queued |
| Wiring by D-label vs GPIO number caused dead parts | Every pin table now lists the XIAO pad first |
| Light strip flickered between an animation and red; errors shown with no phone connected | Workflow engine: one look at a time, problems only while live |

Table 20. Usability issues found and fixed

# 12. Testing and quality assurance

| Area | How it is tested | Result |
| --- | --- | --- |
| Backend | pytest: ingest and alerts, shelf life, thresholds, device auth, GPS, error logging, LLM orchestrator, Model 1 seam, analytics, products, planner, mixed loads | 72 passed |
| Admin panel | ESLint, TypeScript (tsc), next build | Clean |
| Phone app | node --check on every module; browser checks with a simulated Bluetooth device | Clean |
| Firmware | arduino-cli compile --warnings all | 0 warnings |
| Hardware | Bench checks on the real board with the serial console and probe sketches | DHT22 24.5 °C, door open/closed, strip GRB on 11 LEDs |
| End to end | Real backend, real local LLM, scripted shipments | See below |

Table 21. Test coverage

End-to-end results on the running system:

- The Load Planner split apples, pears, lettuce and bananas into 3 compartments.

- An apples + bananas shipment was rejected with reasons (ethylene and temperature).

- An apples + pears shipment was created at −0.9 °C (method: ML, guarded by physics).

- Warm readings were flagged against the setpoint, with per-product shelf life live.

- The AI explanation reached the phone 7 seconds after the alert; the LLM test answered in 1.6 s.

Honest status Verified on hardware: the DHT22 on D8, the reed switch on D4, and the 11-LED strip on D3 in GRB order. Still to verify: the IR LED on D5 with the x test, and AC pairing against the target air conditioner.

# 13. Demonstration plan

## 13.1 Before the demo

- Run python start_frostmind.py on the presentation laptop. Use the venue Wi-Fi or the laptop's hotspot; both work.

- On the phone, accept the certificate at :8000/health and :5500, open the app and use Reset wizard.

- Open the admin panel, then AI Models, then Test LLM, so the model is loaded before the audience is watching.

- Power the box. Keep the sensor in a cool box with an ice pack, so the live view starts at the shelf-life bar rather than an alarm.

- Have the serial console ready as a fallback.

## 13.2 Ten-minute script

| Time | Show | Point to make |
| --- | --- | --- |
| 0:00 | Architecture slide | Sense, act, explain; everything local |
| 1:00 | Power the box: frost bloom, then the blue beacon | No phone, so no alarms: nobody would see them |
| 2:00 | Wizard steps 1-2: pair (handshake on the strip), register | Nothing typed; the device ID comes over Bluetooth |
| 3:00 | Step 3: apples, pears, lettuce, bananas, then Check load (AI) | Model 5 splits them and explains why; Model 4 gives −0.9 °C |
| 4:30 | Create the apples + pears shipment; the strip ripples and ticks 'product' | The box knows the workflow |
| 5:00 | Step 5: live data + Flash the lights (rainbow) | Both directions verified |
| 5:30 | Go live: sweep, then the shelf-life bar | On idle, just the shelf life |
| 6:00 | Take the sensor out / open the door | Coral heartbeat or amber door light; AI explanation on the phone within seconds |
| 7:30 | Admin panel: fleet, shipment page, alerts, Load Planner, AI Models | The manager's view, with measured model scores |
| 9:00 | Close: tests, deployment on any laptop, roadmap | Ready to run anywhere; honest about limits |

Table 22. Demonstration script

## 13.3 Fallbacks

- Bluetooth fails: press d in the serial console to play the whole light-strip workflow, and use the admin panel with the shipment already created.

- The LLM is slow: the phone shows 'AI explanation being written…' and the deterministic alert is already there; explain the 45-second background design.

- Wi-Fi blocks devices from seeing each other: use the laptop's hotspot. The launcher finds the new address by itself.

- The AC cannot be reached: show the IR LED flashing through a phone camera with x.

# 14. Future scope

| Horizon | Feature | Value |
| --- | --- | --- |
| Next | Train Model 1 on FROSTMIND's own logged trips and switch it on | Learned anomaly detection on the live stream; the seam is ready |
| Next | Real-time push (WebSockets) instead of 10-second polling | Alerts appear instantly |
| Next | Map view of trucks (GPS is already recorded with every reading) | Fleet-wide situational awareness |
| Mid | INA219 current sensor, then Model 2 root-cause classification | Tell a compressor fault from an open door or a power cut |
| Mid | Learned correction layer on top of Model 3 using delivery-quality outcomes | Shelf life calibrated to real products |
| Mid | Fine-tune Model 4 and 5 with real delivery outcomes | From expert labels to measured outcomes |
| Mid | Over-the-air firmware updates (ESP32 OTA) | Update boxes without a laptop and cable |
| Mid | Cellular (LTE-M) or LoRaWAN module on the box | No dependence on the driver's phone |
| Long | Cloud deployment: PostgreSQL, TLS domain, Docker, GPU LLM server | Many fleets, many trucks |
| Long | Multi-tenant accounts and roles (driver, manager, auditor) | Sell to more than one company |
| Long | Native mobile app (React Native or Flutter) | iOS support and true background operation |
| Long | Integration with warehouse/ERP systems; GDP/FSMA-style certified reports | Fits into real logistics workflows |
| Long | Rugged enclosure, battery backup, DS18B20 probes per compartment | Field-ready hardware |

Table 23. Roadmap

# 15. Maintenance strategy

FROSTMIND is built so that a small team can keep it healthy: every part reports its own status, models can be retrained with one command, and nothing breaks silently.

## 15.1 Routine tasks

| Area | Task | When | How |
| --- | --- | --- | --- |
| Code | Run all checks before merging | Every change | pytest; npm run lint + build; arduino-cli compile --warnings all |
| Code | Review on a pull request into master | Every feature | GitHub PR |
| Models | Rebuild the catalog and retrain Models 4 and 5 | When the catalog changes, else quarterly | scripts/train_all.py; compare *_metrics.json |
| Models | Check Model 4's σ against real truck data | Monthly | Planner uses each truck's own σ when available |
| Data | Back up the database | Weekly and before upgrades | Timestamped copy of frostmind.db |
| LLM | Check the LLM is reachable | Daily in use | AI Models page, Test LLM, /health |
| Hardware | Compare the DHT22 with a reference thermometer | Monthly | Replace if off by more than 0.5 °C |
| Hardware | Check wiring, strip and IR | Monthly and after moving a box | Console keys g, d, x |
| Security | Rotate the JWT secret and admin password | Before a pilot, then every 90 days | Environment variables |
| Security | Renew the HTTPS certificate | Automatic | Launcher regenerates it under 30 days left |
| Dependencies | Update libraries, then run the full test suite | Quarterly | Pinned versions (esp32 core 3.3.11, IRremoteESP8266 2.9.0) |
| Docs | Update docs with the change they describe | Every change | Same commit as the code |

Table 24. Maintenance schedule

## 15.2 Model governance

- Every model ships with its metrics: each .pkl has a *_metrics.json with its scores, threshold and features, shown on the AI Models page.

- Promotion gate: a retrained model replaces the old one only if its product-disjoint scores are at least as good (e.g. Model 5 ROC-AUC ≥ 0.90, Model 4 within ±0.5 °C ≥ 90 %).

- Guarded serving: Model 4's answer is used only if the physics optimiser agrees to 0.5 °C, otherwise physics wins. If a .pkl is missing the backend falls back to physics and rules instead of crashing.

- Safe activation: Model 1 turns on only when a model trained on FROSTMIND's own data exists; until then it is a documented no-op.

- LLM boundary: the LLM can never act; any failure leaves the deterministic alert intact.

## 15.3 Monitoring and incident response

| Signal | Where | Response |
| --- | --- | --- |
| Backend health, model and LLM status | /health, AI Models page | Restart the launcher or Ollama |
| Device stopped reporting | Fleet page (online state), phone BLE tile | Check power and Bluetooth; phone reconnects automatically |
| IR path alive | 'Last IR command sent' (heartbeat every 5 min) | If stale: check the IR LED with x |
| Readings waiting to sync | Phone 'Pending sync' tile | Check Wi-Fi; nothing is lost, the queue uploads later |
| Errors | Structured backend logs | Each failure logged with context |

Table 25. Monitoring signals

## 15.4 Data and schema changes

Schema changes are additive (ensure_columns() adds new nullable columns on start-up), so upgrading never deletes data. Back up the database before any upgrade. Moving to PostgreSQL is a connection-string change because every query goes through the ORM.

## 15.5 Roles

| Role | Responsible for |
| --- | --- |
| Firmware owner | Firmware changes, flashing, hardware checks, wiring docs |
| Backend and AI owner | API, database, model retraining and promotion, LLM setup |
| Frontend owner | Phone app and admin panel, usability fixes |
| Release owner (rotating) | Runs the checks, merges the PR, updates documentation, backs up the database |

Table 26. Maintenance roles

# 16. Limitations

- Android only: iOS Safari does not support Web Bluetooth.

- The phone must keep the app in the foreground; browsers cannot hold Bluetooth in the background.

- The LAN deployment uses a self-signed certificate and a single SQLite database, and the setup endpoints are open on the local network.

- Model 1 is validated but not live; Model 2 is not built.

- Model 3's constants are illustrative literature values, not certified for a specific product.

- Models 4 and 5 are trained on expert labels and simulated loads; non-produce pairs are handled by rules and marked as extrapolated.

- IR is transmit-only: the box cannot confirm the AC received a command (mitigated by the heartbeat).

- Not certified for regulated pharmaceutical decisions.

# 17. Conclusion

FROSTMIND is a complete, working cold-chain system built on low-cost, off-the-shelf hardware. It senses every 2 seconds, corrects the cooling unit on its own, measures shelf life per product, recommends temperatures for mixed loads, decides which products can travel together, and explains every incident in plain English, all on one laptop with no cloud account.

For Phase 7 the system is deployable on any laptop with one command and usable by a driver through a guided setup and a glanceable light strip. It is documented and tested (72 automated tests), with a clear plan for demonstration, growth and maintenance.

# 18. References

- Gross, K. C., Wang, C. Y. and Saltveit, M. (eds.) (2016). The Commercial Storage of Fruits, Vegetables, and Florist and Nursery Stocks. USDA Agriculture Handbook 66.

- McGregor, B. M. (1989). Tropical Products Transport Handbook. USDA Agriculture Handbook 668 (compatibility groups).

- Engineering ToolBox. Fruits and Vegetables: Optimal Storage Conditions.

- U.S. FDA (2022). Food Code.

- U.S. CDC. Vaccine Storage and Handling Toolkit.

- WHO / European Commission. Good Distribution Practice guidelines.

- Labuza, T. P. (1984). Application of chemical kinetics to deterioration of foods. Journal of Chemical Education, 61(4).

- Liu, F. T., Ting, K. M. and Zhou, Z.-H. (2008). Isolation Forest. IEEE ICDM.

- Lavin, A. and Ahmad, S. (2015). Evaluating real-time anomaly detection algorithms: the Numenta Anomaly Benchmark. IEEE ICMLA.

- Abhiraman et al. (2023). Vaccine-refrigeration fault simulation with CNNs. International Journal of Refrigeration.

- Brélaz, D. (1979). New methods to color the vertices of a graph (DSatur). Communications of the ACM, 22(4).

- Pedregosa, F. et al. (2011). Scikit-learn: machine learning in Python. JMLR, 12.

- Espressif Systems. ESP32-S3 Technical Reference Manual; Seeed Studio XIAO ESP32S3 wiki.

- W3C Web Bluetooth Community Group. Web Bluetooth specification.

- FastAPI, Next.js, Ollama, Gemma and IRremoteESP8266 project documentation.

# Appendix A. Repository structure

| Path | Contents |
| --- | --- |
| start_frostmind.py | One-command launcher (servers, certificate, addresses) |
| firmware/frostmind_main/ | Device firmware and its README (BLE contract, light strip, flashing) |
| phone-bridge/ | Phone PWA: js/ (wizard, dashboard, ble, api, sync, idb, geo), css/, service worker, serve_https.py |
| backend/app/ | FastAPI app: routers/, models.py, schemas.py, ml.py, load_planner.py, llm_orchestrator.py |
| backend/tests/ | 72 automated tests |
| web/src/ | Admin panel: app/ (pages), components/, lib/ |
| scripts/ | Data fetching, cleaning, training (train_all.py), esp_console.py |
| data/ | Datasets and the product catalog with its cleaning report |
| models/ | Trained models and their metrics (the .pkl files are built locally) |
| docs/ | Figures and the document converter |
| Phase7_Submission/ | This document, the presentation, figures and their generators |

Table 27. Repository structure

# Appendix B. API endpoints

| Method | Path | Auth | Purpose |
| --- | --- | --- | --- |
| POST | /auth/login · GET /auth/me | - · JWT | Admin login and profile |
| POST | /devices/register | open | Register a device, get its key |
| GET/PATCH | /devices, /devices/{id} | open | List and update devices |
| POST/GET | /devices/{id}/pairing/start, /status | open | AC pairing state from the phone |
| GET/POST/PATCH | /trucks, /trucks/{id} | open | Trucks |
| POST | /shipments | open | Create a shipment (category or products, Model 5 check, Model 4 setpoint) |
| GET/PATCH | /shipments, /shipments/{id} | JWT | Fleet list, detail, status |
| GET | /shipments/{id}/readings, /alerts | JWT | History |
| GET | /shipments/{id}/live | Device key | Phone AI feed |
| GET / POST | /shipments/{id}/report · /compliance_report | JWT | CSV export · LLM report |
| POST | /ingest | Device key | Upload readings |
| GET/PATCH | /alerts, /alerts/{id} | JWT | Alerts and acknowledgement |
| GET | /analytics/fleet | JWT | Fleet trends |
| GET | /products | open | 155-product catalog |
| POST | /planner/plan | open | Compartments, setpoints, compatibility |
| GET · POST | /ai/status · /ai/llm/test | open · JWT | AI component status · LLM round trip |
| GET | /health | open | Server, model and LLM status |

Table 28. API endpoints (interactive docs at /docs)

# Appendix C. Command cheat sheet

# run everything

.venv\Scripts\python start_frostmind.py

# tests and checks

cd backend && ..\.venv\Scripts\python -m pytest

cd web && npm run lint && npm run build

arduino-cli compile --warnings all --fqbn esp32:esp32:esp32s3:CDCOnBoot=cdc ^

firmware/frostmind_main

# retrain models

.venv\Scripts\python scripts/train_all.py

# device console (keys: x IR test, g strip colours, d strip demo, a auto-pair)

.venv\Scripts\python scripts/esp_console.py

# Appendix D. Glossary

| Term | Meaning |
| --- | --- |
| Cold chain | The temperature-controlled supply chain for perishable goods |
| Reefer | A refrigerated truck or container unit |
| Excursion | A period outside the allowed temperature range |
| Setpoint | The temperature a cooling unit is told to hold |
| Q10 | How many times faster a product decays for a 10 °C rise |
| Ethylene | A ripening gas some fruits emit that damages sensitive produce |
| BLE | Bluetooth Low Energy |
| PWA | Progressive Web App: a web page that installs and works offline like an app |
| RMT | The ESP32's remote-control peripheral, used here to time the LED strip's data |
| MAE / ROC-AUC / MCC | Mean absolute error; area under the ROC curve; Matthews correlation coefficient |
| DSatur | A graph-colouring algorithm, used to split incompatible products into compartments |
| LLM | Large language model; here a small quantised model running locally |

Table 29. Glossary